# DGX Log Utility v2.1.2

# User Guide

#### New in v2.1.2 – Now has the ability to gather debug from Legacy DGXs using the RS232 port

# DGX 100 Series Debug Options:

#### FTP - Uses specified IP to retrieve the historical Log files created each time the DGX reboots. If “Delete DGX Log Files” is checked, the old files will be deleted after running the DGX Log Utility

#### Shell – Uses specified IP to capture the current system settings and splash screen data

#### Master - Uses the specified IP to capture the current settings and logs

#### Archive - Zips all debug files into a timestamped zip file

Delete DGX Log Files - This removes all log files after capturing and archiving them in case there are some with invalid/“future” timestamps that aren’t allowing new log files to be archived. It maintains the last 30 log files by date. A new file is created each reboot.

IMPORTANT:  You must UNZIP the DGX Log Utility to its own folder prior to launching it so it can create some necessary files.

The DGX Log Utility defaults to opening in Telnet mode and there are two ways to connect to the DGX 100 systems.

## Connect via the Private ICS LAN (Default Mode)

- Connect your computer to the ICS LAN port of the DGX CPU

  - Wait for your computer to get a DHCP address from the DGX i.e. 198.18.36.XXX

- Launch the DGX Log Utility

- The default Username and Password should work for most users unless they have be changed

- Do Not Change the default IP settings for the ICSLAN setup (These IP are hard coded to the DGX CPU and Master)

- Click the Get Logs button

- The program will make telnet / ftp connections to the DGX to capture and download the logs

- It will also save the logs and add them to a zip file if the Archive checkbox is checked

- When done, a folder with the logs will open and you can review the files or send the zipped file to Tech Support

## Connect via the Public LAN

- In order to connect to the DGX Shell the Public LAN – an external telnet interface needs to be enabled

  - Please see Appendix D of the DGX Hardware Reference for details

- The basic commands are as follows for a DHCP network

Setup

  - send_command 5002:1:1, "'IFCONFIG-1, UP, D, hn=DGX_Telnet'"

  - Confirm

  - Enable notifications on 5002

  - send_command 5002:1:1, "'?IFCONFIG-1'"

  - Response: IFCONFIG-1, UP, D, ip=10.103.4.155, gw=10.103.4.1, mask=255.255.255.0, hn=DGX_Telnet

- Launch the DGX Log Utility

- Change the FTP and Shell IP addresses to match the address that the DGX received and was verified

- Change the Master IP address to the external IP of the DGX Master

- Click the Get Logs button

- The program will make telnet / ftp connections to the DGX to capture and download the logs

- It will also save the logs and add them to a zip file if the Archive checkbox is checked

- When done, a folder with the logs will open and you can review the files or send the zipped file to Tech Support

# Legacy DGX Debug Options:

## Connect to the RS232 Serial Debug port

- Connect your computer to the DB9 connector of the DGX

- Launch the DGX Log Utility

- Select File then Serial Debug

- The interface will change to show the Serial port options

- Select the COM from the list

- The baud is fixed at 9600 (for use with Legacy DGX systems.  If using a DGX 100 use one of the Ethernet connections)

Note: the Username and Password are not needed

- Select “Get Logs”

- The program will make the serial connection to the DGX and capture the logs

- It will also save the logs and add them to a zip file if the Archive checkbox is checked

- When done, a folder with the logs will open and you can review the files or send to Tech Support

## Notes

The program creates three commands files (master_commands.txt, shell_commands.txt, & serial_commands.txt) in the active directory when it is first run. If you need to change the master or shell commands that are sent, you can edit these files with one command per line and no trailing lines at the end of the file.  If you need to change the serial commands, follow the syntax given in the txt file for shell and bcs commands.   To reset these files, just delete them and they will be recreated.

The program also creates a configuration file dgx_log_utility.conf  This file stores any changes you make to the IP addresses or file names. To reset all settings, just delete this file, and the defaults will be recreated the next time the program is run.

A big shout out to Jim Maciejewsky [Madison AV (AU)] who used his great Python skills to help develop this DGX diagnostics tool.