# DGX Log Utility v2.1.2

# User Guide

#### The DGX Log Utility makes gathering logs from DGX systems easy.

#### New in v2.1.2 – Now has the ability to gather debug from Legacy DGXs using the RS232 port

# DGX 100 Series Debug Options:

#### FTP - Uses specified IP to retrieve the historical Log files created each time the DGX reboots. If “Delete DGX Log Files” is checked, the old files will be deleted after running the DGX Log Utility

#### Shell – Uses specified IP to capture the current system settings and splash screen data

#### Master - Uses the specified IP to capture the current settings and logs

#### Archive - Zips all debug files into a timestamped zip file

-----------------------------------------------------------------------------------------------------

The DGX Log Utility defaults to opening in Telnet mode and there are two ways to connect to the DGX 100 systems.

## Connect via the Private ICS LAN (Default Mode)

- Connect your computer to the ICS LAN port of the DGX CPU

  - Wait for your computer to get a DHCP address from the DGX i.e. 198.18.36.XXX

- Launch the DGX Log Utility

- The default Username and Password should work for most users unless they have be changed

- Click the Get Logs button

- The program will make telnet / ftp connections to the DGX to capture and download the logs

- It will also save the logs and add them to a zip file if the Archive checkbox is checked

- When done, a folder with the logs will open and you can review the files or send to Tech Support

## Connect via the Public LAN

- In order to connect to the DGX Shell the Public LAN – an external telnet interface needs to be enabled

  - Please see Appendix D of the DGX Hardware Reference for details

- The basic commands are as follows for a DHCP network

Setup

  - send_command 5002:1:1, "'IFCONFIG-1, UP, D, hn=DGX_Telnet'"

  - Confirm

  - Enable notifications on 5002

  - send_command 5002:1:1, "'?IFCONFIG-1'"

  - Response: IFCONFIG-1, UP, D, ip=10.103.4.155, gw=10.103.4.1, mask=255.255.255.0, hn=DGX_Telnet

- Launch the DGX Log Utility

- Change the FTP and Shell IP addresses to match the address that the DGX received and was verified

- Change the Master IP address to the external IP of the DGX Master

- Click the Get Logs button

- The program will make telnet / ftp connections to the DGX to capture and download the logs

- It will also save the logs and add them to a zip file if the Archive checkbox is checked

- When done, a folder with the logs will open and you can review the files or send to Tech Support

# Legacy DGX Debug Options:

#### Archive - Zips all debug files into a timestamped zip file.  The Serial Debug logs are limited and much smaller than for the DGX 100 Series so archiving really isn’t necessary. But if you uncheck it for Serial Debug, remember to check it again if capture DGX 100 logs.

## Connect to the RS232 Serial Debug port

- Connect your computer to the DB9 connector of the DGX

- Launch the DGX Log Utility

- Select File then Serial Debug

- The interface will change to show the Serial port options

- Select the COM from the list

- The baud is fixed at 9600 (for use with Legacy DGX systems.  If using a DGX 100 use one of the Ethernet connections)

Note: the Username and Password are not needed

- Select “Get Logs”

- The program will make the serial connection to the DGX and capture the logs

- It will also save the logs and add them to a zip file if the Archive checkbox is checked

- When done, a folder with the logs will open and you can review the files or send to Tech Support

## Notes

The program creates three commands files (master_commands.txt, shell_commands.txt, & serial_commands.txt) in the active directory when it is first run. If you need to change the master or shell commands that are sent, you can edit these files with one command per line and no trailing lines at the end of the file.  If you need to change the serial commands, follow the syntax given in the txt file for shell and bcs commands.   To reset these files, just delete them and they will be recreated.

The program also creates a configuration file dgx_log_utility.conf  This file stores any changes you make to the IP addresses or file names. To reset all settings, just delete this file, and the defaults will be recreated the next time the program is run.